CPE301 – SPRING 2019

# Design Assignment 3A

Student Name: Geovanni Portillo

Student #: 8000603824

Student Email: portig1@unlv.nevada.edu

Primary Github address: https://github.com/portig1/submissions_E

Directory: submissions_E/DA/LAB3A/

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7

ATmega328PB Xplained mini

- INITIAL CODE OF TASK 1

#define F_CPU 16000000UL

#include <avr/io.h>

#include <util/delay.h>

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

//Declaration of our functions

void USART_init(void);

unsigned char USART_receive(void);

void USART_send( unsigned char data);

void USART_putstring(char* StringPtr);

char String[] = "Hello world!!";  //String[] is in fact an array but when we put the text between the " " symbols the compiler threats it as a String and automatically puts the null termination character in the end of the text

int main(void) {

USART_init();        //Call the USART initialization code

while (1) {      //Infinite loop

USART_putstring(String);    //Pass the string to the USART_putstring function and sends it over the serial

_delay_ms(5000);        //Delay for 5 seconds so it will re-send the string every 5 seconds

}

return 0;

}

void USART_init(void) {

UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8);

UBRR0L = (uint8_t)(BAUD_PRESCALLER);

UCSR0B = (1 << RXEN0) | (1 << TXEN0);

UCSR0C = (3 << UCSZ00);

}

unsigned char USART_receive(void) {

while (!(UCSR0A & (1 << RXC0)));

return UDR0;

}

void USART_send( unsigned char data) {

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = data;

}

void USART_putstring(char* StringPtr) {

while (*StringPtr != 0x00) {

USART_send(*StringPtr);

StringPtr++;

}

}

- MODIFIED CODE OF TASK1

#include <stdio.h>	//included for snprintf

#include <stdlib.h> //included for rand

#define F_CPU 16000000UL

#include <avr/io.h>

#include <avr/interrupt.h>

#define BAUDRATE 9600

#define BAUD_PRESCALLER (((F_CPU / (BAUDRATE * 16UL))) - 1)

//Declaration of our functions

void USART_init(void);

unsigned char USART_receive(void);

void USART_send( unsigned char data);

void USART_putstring(char* StringPtr);

//Declaration of global variables

volatile float randomFlt;

volatile int randomInt;

char String1[] = "The random integer is ";  //String[] is in fact an array but when we put the text between the " " symbols the compiler threats it as a String and automatically puts the null termination character in the end of the text

char String2[] = ", and the random float is ";

char randomIntString[20];

char randomFltString[20];

int main(void) {

USART_init();        //Call the USART initialization code

TIMSK1 = (1 << OCIE1A);

sei();

OCR1A = 62499; //Using TCNT = clk*delay/prescaler - 1 to find OCR1A given clk = 16MHz, OCR1A was calculated to 62,499

TCCR1A = 0; // COM1A/B Normal Operation, OC1A/B Disconnected

TCCR1B = (1 << WGM12) | (1 << CS12); //WGM CTC Mode, Prescaler = 256

while (1) {      //Infinite loop

}

return 0;

}

void USART_init(void) {

UBRR0H = (uint8_t)(BAUD_PRESCALLER >> 8);

UBRR0L = (uint8_t)(BAUD_PRESCALLER);

UCSR0B = (1 << RXEN0) | (1 << TXEN0);

UCSR0C = (3 << UCSZ00);

}

unsigned char USART_receive(void) {

while (!(UCSR0A & (1 << RXC0)));

return UDR0;

}

void USART_send( unsigned char data) {

while (!(UCSR0A & (1 << UDRE0)));

UDR0 = data;

}

void USART_putstring(char* StringPtr) {

while (*StringPtr != 0x00) {

USART_send(*StringPtr);

StringPtr++;

}

}

ISR (TIMER1_COMPA_vect) // timer1 compare interrupt

{

randomInt = rand();

randomFlt = (randomInt / 13); //Generate random integer then divide by 13 to get a different floating point number

snprintf(randomIntString, sizeof(randomIntString), "%d", randomInt);

snprintf(randomFltString, sizeof(randomFltString), "%f\r\n", randomFlt); //convert numbers to strings

USART_putstring(String1);    //Pass the string to the USART_putstring function and sends it over the serial

USART_putstring(randomIntString);

USART_putstring(String2);

USART_putstring(randomFltString);

}

- SCHEMATICS

No schematic needed, no external components used

- SCREENSHOTS OF EACH TASK OUTPUT (ATMEL STUDIO OUTPUT)

Not asked for in instructions

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Board setup for Task 1

- VIDEO LINKS OF EACH DEMO

https://youtu.be/0QmDsDwE4xA

- GITHUB LINK OF THIS DA

https://github.com/portig1/submissions_E/tree/master/DA/LAB3A

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Geovanni Portillo