CPE301 – SPRING 2019

# Design Assignment 3B

Student Name: Geovanni Portillo

Student #: 8000603824

Student Email: portig1@unlv.nevada.edu

Primary Github address: https://github.com/portig1/submissions_E

Directory: submissions_E/DA/LAB3B/

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7

ATmega328PB Xplained mini

LM35 Schematic

Arduino Shield U5 Schematic

- INITIAL CODE OF TASK 1 (Module8B_3 from website files)

#define F_CPU 16000000UL

#define BAUD_RATE 9600

#include <avr/io.h>

#include <util/delay.h>

void usart_init ();

void usart_send (unsigned char ch);

int main (void)

{

usart_init ();

/** Setup and enable ADC **/

ADMUX = (0<<REFS1)|    // Reference Selection Bits

(1<<REFS0)|    // AVcc - external cap at AREF

(0<<ADLAR)|    // ADC Left Adjust Result

(1<<MUX2)|     // Analog Channel Selection Bits

(0<<MUX1)|     // ADC4 (PC4 PIN27)

(0<<MUX0);

ADCSRA = (1<<ADEN)|    // ADC ENable

(0<<ADSC)|     // ADC Start Conversion

(0<<ADATE)|    // ADC Auto Trigger Enable

(0<<ADIF)|     // ADC Interrupt Flag

(0<<ADIE)|     // ADC Interrupt Enable

(1<<ADPS2)|    // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

while (1)

{

ADCSRA|=(1<<ADSC);	//start conversion

while((ADCSRA&(1<<ADIF))==0);//wait for conversion to finish

ADCSRA |= (1<<ADIF);

int a = ADCL;

a = a | (ADCH<<8);

a = (a/1024.0) * 5000/10;

usart_send((a/100)+'0');

a = a % 100;

usart_send((a/10)+'0');

a = a % 10;

usart_send((a)+'0');

usart_send('\r');

_delay_ms(100);

}

return 0;

}

void usart_init (void)

{

UCSR0B = (1<<TXEN0);

UCSR0C = (1<< UCSZ01)|(1<<UCSZ00);

UBRR0L = F_CPU/16/BAUD_RATE-1;

}

void usart_send (unsigned char ch)

{

while (! (UCSR0A & (1<<UDRE0))); 	//wait until UDR0 is empty

UDR0 = ch;							//transmit ch

}

void usart_print(char* str)

{

int i = 0;

while(str[i] != 0)

usart_send(str[i]);

}

- MODIFIED CODE OF TASK1

#define F_CPU 16000000UL

#define BAUD_RATE 9600

#include <avr/io.h>

#include <avr/interrupt.h>

#include <stdio.h>

void usart_init ();

void USART_send( unsigned char data);

void USART_putstring(char* StringPtr);

int main (void)

{

usart_init ();

sei();

TIMSK1 = (1 << OCIE1A);

OCR1A = 62499; //Using TCNT = clk*delay/prescaler - 1 to find OCR1A given clk = 16MHz, OCR1A was calculated to 62,499

TCCR1A = 0; // COM1A/B Normal Operation, OC1A/B Disconnected

TCCR1B = (1 << WGM12) | (1 << CS12); //WGM CTC Mode, Prescaler = 256

/** Setup and enable ADC **/

ADMUX = (0<<REFS1)|    // Reference Selection Bits

(1<<REFS0)|    // AVcc - external cap at AREF

(0<<ADLAR)|    // ADC Left Adjust Result

(1<<MUX2)|     // Analog Channel Selection Bits

(0<<MUX1)|     // ADC4 (PC4 PIN27)

(0<<MUX0);

ADCSRA = (1<<ADEN)|    // ADC Enable

(0<<ADSC)|     // ADC Start Conversion

(0<<ADATE)|    // ADC Auto Trigger Enable

(0<<ADIF)|     // ADC Interrupt Flag

(0<<ADIE)|     // ADC Interrupt Enable

(1<<ADPS2)|    // ADC Prescaler Select Bits

(0<<ADPS1)|

(1<<ADPS0);

while (1)

{

}

return 0;

}

ISR(TIMER1_COMPA_vect)

{

ADCSRA|=(1<<ADSC);	//start conversion

while((ADCSRA&(1<<ADIF))==0);//wait for conversion to finish

ADCSRA |= (1<<ADIF);

char output[20];

int tempC = ADCL;

tempC = tempC | (ADCH<<8);

tempC = (tempC/1024.0) * 5000/10;

snprintf(output, sizeof(output), "%d\r\n", tempC);

USART_putstring(output);

}

void usart_init (void)

{

UCSR0B = (1<<TXEN0);

UCSR0C = (1<< UCSZ01)|(1<<UCSZ00);

UBRR0L = F_CPU/16/BAUD_RATE-1;

}

void USART_send( unsigned char data) {

while (!(UCSR0A & (1 << UDRE0))); //wait until UDR0 is empty

UDR0 = data;					 //transmit ch

}

void USART_putstring(char* StringPtr) {

while (*StringPtr != 0x00) {

USART_send(*StringPtr);

StringPtr++;

}

}

- SCHEMATICS

- SCREENSHOTS OF EACH TASK OUTPUT

Task 1/Task 2 output in Serial Plot

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

Board setup for Task 1/Task 2

- VIDEO LINKS OF EACH DEMO

https://youtu.be/TBz9H7icXFQ

- GITHUB LINK OF THIS DA

https://github.com/portig1/submissions_E/tree/master/DA/LAB3B

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Geovanni Portillo