CPE301 – SPRING 2019

# Design Assignment 5

Student Name: Geovanni Portillo

Student #: 8000603824

Student Email: portig1@unlv.nevada.edu

Primary Github address: https://github.com/portig1/submissions_E

Directory: submissions_E/DA/LAB5/

Submit the following for all Labs:

- In the document, for each task submit the modified or included code (only) with highlights and justifications of the modifications. Also, include the comments.

- Use the previously create a Github repository with a random name (no CPE/301, Lastname, Firstname). Place all labs under the root folder ESD301/DA, sub-folder named LABXX, with one document and one video link file for each lab, place modified asm/c files named as LabXX-TYY.asm/c.

- If multiple asm/c files or other libraries are used, create a folder LabXX-TYY and place these files inside the folder.

- The folder should have a) Word document (see template), b) source code file(s) and other include files, c) text file with youtube video links (see template).

- COMPONENTS LIST AND CONNECTION BLOCK DIAGRAM w/ PINS

Atmel Studio 7

ATmega328PB Xplained mini

LM35 Schematic

Arduino Shield U5 Schematic

NRF2424L01 Schematic

- MODIFIED CODE OF TASK 1

/*

* Task_1.c

*

* Created: 4/22/2019 2:28:58 PM

* Author : gausp

*/

//Worked with Robert Sander and Rocky Gonzalez trying to send and

//receive data using the ATmega328 and NRF24L01 module.

//The code remains mostly unchanged from the files at https://github.com/venki666/cpe301Demo/tree/master/Libraries/NRF24L01_LIB/NRF24L01_BM_RX

//The spi and nrf24l01 codes required changes to the registers such as SPSR being changed to SPSR0 to use the 328PB

//We noticed that the configuration would be properly setup on the 328P but for the 328PB the configuration would be done wrong

//For example, RF_CH is supposed to be 0x74 but for the PB it gets set to 0x30.

//We tested this by only swapping out the 328P with the 328PB (only one of use had a 328P), the only changes for the code

//were to revert the register name changes previously mentioned.

//The datasheet for the 328PB showed that the SPI pins for the 328P pins such as MOSI

//just become known as MOSI0 but are otherwise the same so we ruled out that as being

//the issue. We also checked the connections several times.

// 	MIT License

//

// 	Copyright (c) 2018 Helvijs Adams

//

// 	Permission is hereby granted, free of charge, to any person obtaining a copy

// 	of this software and associated documentation files (the "Software"), to deal

// 	in the Software without restriction, including without limitation the rights

// 	to use, copy, modify, merge, publish, distribute, sublicense, and/or sell

// 	copies of the Software, and to permit persons to whom the Software is

// 	furnished to do so, subject to the following conditions:

//

// 	The above copyright notice and this permission notice shall be included in all

// 	copies or substantial portions of the Software.

//

// 	THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR

// 	IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY,

// 	FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT. IN NO EVENT SHALL THE

// 	AUTHORS OR COPYRIGHT HOLDERS BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER

// 	LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM,

// 	OUT OF OR IN CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE

// 	SOFTWARE.

//

//	Software was tested on ATmega328P and ATmega328PB (PB needs few changes in SPI)

//	RF module software was tested on - cheap nRF24L01+ from China

//	All the relevant settings are defined in nrf24l01.c file

//	Some features will be added later, at this moment it is bare minimum to send/receive

//

//	Set clock frequency

#ifndef F_CPU

#define F_CPU 16000000UL

#endif

#include <avr/io.h>

#include <util/delay.h>

#include <avr/interrupt.h>

#include <stdbool.h>

#include <stdio.h>

#include <string.h>

//	Set up UART for printf();

#ifndef BAUD

#define BAUD 9600

#endif

#include "STDIO_UART.h"

//	Include nRF24L01+ library

#include "nrf24l01.h"

#include "nrf24l01-mnemonics.h"

#include "spi.h"

void print_config(void);

//	Used in IRQ ISR

volatile bool message_received = false;

volatile bool status = false;

//setup ADC

void adc_init();

int adc_getValue();

int main(void)

{

//	Set cliche message to send (message cannot exceed 32 characters)

char tx_message[32];				// Define string array

strcpy(tx_message,"Hello from the Other Side!");	// Copy string into array

//	Initialize UART

uart_init();

//	Initialize ADC and test

adc_init();	//Set up ADC

printf("Testing ADC, Temperature is: %d Celsius\n\n",adc_getValue());

//	Initialize nRF24L01+ and print configuration info

nrf24_init();

print_config();

//	Start listening to incoming messages

nrf24_start_listening();

status = nrf24_send_message(tx_message);

if (status == true) printf("Connection Established\n");

while (1)

{

if (message_received)

{

//	Message received, print it

message_received = false;

printf("Received message: %s\n",nrf24_read_message());

//	Send message as response

_delay_ms(500);

//Perform ADC and store in tx_message

snprintf(tx_message, sizeof(tx_message), "Temp value from Geo: %d Celsius", adc_getValue());

status = nrf24_send_message(tx_message);

if (status == true) printf("Message Transmitted\n");

}

}

}

//	Interrupt on IRQ pin

ISR(INT0_vect)

{

message_received = true;

}

void print_config(void)

{

uint8_t data;

printf("Startup successful\n\n nRF24L01+ configured as:\n");

printf("-------------------------------------------\n");

nrf24_read(CONFIG,&data,1);

printf("CONFIG		0x%x\n",data);

nrf24_read(EN_AA,&data,1);

printf("EN_AA			0x%x\n",data);

nrf24_read(EN_RXADDR,&data,1);

printf("EN_RXADDR		0x%x\n",data);

nrf24_read(SETUP_RETR,&data,1);

printf("SETUP_RETR		0x%x\n",data);

nrf24_read(RF_CH,&data,1);

printf("RF_CH			0x%x\n",data);

nrf24_read(RF_SETUP,&data,1);

printf("RF_SETUP		0x%x\n",data);

nrf24_read(STATUS,&data,1);

printf("STATUS		0x%x\n",data);

nrf24_read(FEATURE,&data,1);

printf("FEATURE		0x%x\n",data);

printf("-------------------------------------------\n\n");

}

void adc_init()

{

/** Setup and enable ADC **/

ADMUX =

(0<<REFS1)|    // Reference Selection Bits

(1<<REFS0)|    // AVcc - external cap at AREF

(0<<ADLAR)|    // ADC Left Adjust Result

(1<<MUX2)|     // Analog Channel Selection Bits

(0<<MUX1)|     // ADC4 (PC4 PIN27)

(0<<MUX0);

ADCSRA =

(1<<ADEN)|	// ADC Enable

(0<<ADSC)|	// ADC Start Conversion

(0<<ADATE)|	// ADC Auto Trigger Enable

(0<<ADIF)|	// ADC Interrupt Flag

(0<<ADIE)|	// ADC Interrupt Enable

(1<<ADPS2)|	// ADC Prescaler Select Bits

(0<<ADPS1)|	// CLK/32

(1<<ADPS0);

}

int adc_getValue()

{

ADCSRA|=(1<<ADSC);	//start conversion

while((ADCSRA&(1<<ADIF))==0);//wait for conversion to finish

ADCSRA |= (1<<ADIF);

int adcValue = ADCL;

adcValue = adcValue | (ADCH<<8);

adcValue = (adcValue/1024.0) * 5000/10;	//finish properly formatting tempC value

return adcValue;

}

- SCHEMATICS

Task 1 Schematic

- SCREENSHOTS OF EACH TASK OUTPUT

Task 1:

- SCREENSHOT OF EACH DEMO (BOARD SETUP)

- GITHUB LINK OF THIS DA

https://github.com/portig1/submissions_E/tree/master/DA/LAB5

Student Academic Misconduct Policy

http://studentconduct.unlv.edu/misconduct/policy.html

“This assignment submission is my own, original work”.

Geovanni Portillo